ENIGMA DTI Protocols

Preprocessing           ENIGMA-DTI Processing         GWAS Analysis

Preprocessing

By preprocessing, we are referring to converting your images from the raw DICOM to FA images for each subject and quality controlling along the way to remove scans with abnormalities and artifacts. If you have already extracted good quality FA and diffusivity measures from your DTI images, and have quality checked them, please let us know how you have done this, email support.enigmaDTI@ini.usc.edu and we will advise on whether you should skip to the QC.

There can be several ways to pre-process your diffusion weighted data in order to maximize the quality and efficiency of your processing.

We will therefore not require a specific protocol to be followed with any particular software, as long as the appropriate steps are performed. This will allow maximal integration with current pipelines and ensure optimal processing per site if available, and allow sites to:

process data efficiently with respect to acquisition parameters (eg., do not spend time on HARDI-specific algorithms if you only have 6/12 directions collected)

- take advantage of your scanning protocols

- if you know you can calculate FA more robustly using one of many alternate methods, go for it!

- maximize the quality for your scans (denoising/ removing artifacts etc.)

- keep things in line with current/future projects, and non ENIGMA-related investigations you are working on.

If you have FA measures maps calculated and registered already, we can work with you to include them into the Pipeline rather than to re-run everything from the start. Therefore, if you have discovered workflows and methods that fit your data well to best improve SNR, this would be ideal.

If you have already processed your data, please email support.enigmaDTI@ini.usc.edu to let us know your processing workflow. Also if you would like to update this page with any particulars for your methods, please let us know and we would be happy to work in additional options.

For those that have yet to process DTI data, various suggestions are outlined here.

A basic series of steps are as follows:

NOTE: most of this can be done in multiple ways depending on your data.

Please do not hesitate to contact us for support.

- Convert DICOM images to DWI set and T1-weighted set and other data acquired.

- Determine how your DWI set(s) are organized

- How many many acquisitions do you have? Multiple acquisitions can be merged for optimal signal-to-noise ratio.

- How many b0s do you have and where are they with respect to the full series? (Often the b0 image(s) is/are the first volumes in the DWI set)

- If you have multiple b0, were they acquired with the same encoding gradient? If so, slight variations in processing will be needed.

- Correct for Eddy Current distortions, movement using affine registration.

- A convenient option for this is FSL’s “eddy_correct” command.

- You can use this script ** http://enigma.ini.usc.edu/wp-content/uploads/DTI_Protocols/fdt_rotate_bvecs.sh** to rotate your bvec files (gradient directions) after using FSL’s “eddy_correct” command. (You can use the output of the file from now on and to create your FA images (e.g. as input to dtifit))

- Create a mask for your data.

- FSL’s bet2 offers a solution that is quite robust for many datasets.

- Correct for EPI induced susceptibility artifacts — this is particularly an issue at higher magnetic fields.

- If you have two opposing b0s and a sufficient amount of diffusion directions obtained, you may use FSL’s TOPUP and EDDY for distortion correction.

- If a fieldmap has been collected along with your data, FSL’s FUGUE tool may help compensate for the distortions correction.

- Alternatively, a subject’s DWI images can be adjusted through high dimensional warping of the b0 to a high-resolution structural (T1- or T2- weighted) image of the same subject not acquired using EPI. This requires multiple steps:

- Make sure skull-stripping has been performed on both b0 and T1-weighted scans.

- Make sure T1-weighted scans have undergone inhomogeneity (NU) correction.

- Make sure T1-weighted scans and the DWI are aligned!! Check for L/R flipping!!

- Linear registration of b0 of DWI and T1-weighted reference together. **Due to differences in resolution and further registrations needed throughout the workflow we recommend initially aligning the the T1-weighted scans to ICBM space (which is the space of the ENIGMA-DTI template), then using a linear registration (with NO sheering parameters) to align your b0 maps to their respective T1-weighted scans in ICBM space**

- If using FSL’s flirt for linear registration, sheering can be avoided by manually setting the degrees of freedom (default 12) to 9 (flirt -in b0.nii.gz -out b02T1w.nii.gz -df 9 -ref T1w.nii.gz)

- Once images are in the same space and linearly alight (visually check this!), you can perform non-linear registrations to remove the distortion from the b0.

- Some possible tools include ANTS , DTI-TK, or BrainSuite.

- The deformation fields from the warping should then be applied to all volumes in the DWI.

- Calculate tensors (this can be done in multiple ways depending on your data).

- Most tools will also output FA, MD, and eigenvalue and vector maps simultaneously.

- FSL’s ‘dtifit’ command is an acceptable and convenient option. It uses least-square fitting to determine the tensor and will output FA and V1 (primary eigenvector) needed for future analyses.

Preprocessing Quality Control

Protocol for FA and vector alignment QC analysis for ENIGMA-DTI

The following steps will allow you to visualize your raw FA images before registration to the ENIGMA-DTI template, and to see if your principle direction vectors are appropriately aligned to the white matter tracts.

These protocols are offered with an unlimited license and without warranty. However, if you find these protocols useful in your research, please provide a link to the ENIGMA website in your work: www.enigma.ini.usc.edu

Highlighted portions of the instructions may require you to make changes so that the

commands work on your system and data.

Instructions

Prerequisites

• 	Matlab installed http://www.mathworks.com/products/matlab/

• 	Diffusion-weighted images preprocessed using FSL’s DTIFIT

(http://fsl.fmrib.ox.ac.uk/fsl/fsl4.0/fdt/fdt_dtifit.html) or equivalent. This requires the creation of FA maps and eigenvectors comprising of three volumes, the first being the x-component of the eigenvector, the second being the y-component and the third being the z-component.

Step 1 – Download the utility packages

Download the Matlab scripts package for Step 3:

http://enigma.ini.usc.edu/wp-content/uploads/DTI_Protocols/enigmaDTI_QC.zip

Download the script to build the QC webpage for Step 4:

• Linux: http://enigma.ini.usc.edu/wp-

content/uploads/DTI_Protocols/make_enigmaDTI_FA_V1_QC_webpage.sh

• Mac: http://enigma.ini.usc.edu/wp-

content/uploads/DTI_Protocols/make_enigmaDTI_FA_V1_QC_webpage_mac.sh

Step 2 – Build a text file defining the location of subject files

Create a three column tab-delimited text file (e.g. Subject_Path_Info.txt):

• subjectID: subject ID

• FAimage: full path to original FA image.

• V1image: full path to original V1 image. This is a 4D volume that represents the

primary eigenvector of the diffusion tensors at every voxel (i.e. x-component of the eigenvector).

subjectID				 FAimage 				V1image

USC_01				 /path/to/FA/USC_01_FA.nii.gz 		/path/to/V1/USC_01_V1.nii.gz

USC_02 				/path/to/FA/USC_02_FA.nii.gz 		/path/to/V1/USC_02_V1.nii.gz

USC_03 				/path/to/FA/USC_03_FA.nii.gz 		/path/to/V1/USC_03_V1.nii.gz

Step 3 – Run Matlab script to make QC images

Unzip the Matlab scripts from Step 1 and change directories to that folder with the required Matlab *.m scripts. For simplicity, we assume you are working on a Linux machine with the base directory /enigmaDTI/QC_ENIGMA/.

Make a directory to store all of the QC output:

mkdir /enigmaDTI/QC_ENIGMA/QC_FA_V1/

Start Matlab:

/usr/local/matlab/bin/matlab

Next we will run the func_QC_enigmaDTI_FA_V1.m script that reads the Subject_Path_Info.txt file to create subdirectories in a specified output_directory for each individual subjectID, then create an axial, coronal and sagittal image of the FA_image with vectors from the V1_image overlaid on top. The threshold (0 to ~0.3, default 0.2) overlay the V1 information only on voxels with FA of the specified threshold or greater. Increasing the threshold above 0.1 will run the script faster and is recommended for groups with many subjects.

In the Matlab command window paste and run:

TXTfile='/enigmaDTI/QC_ENIGMA/Subject_Path_Info.txt';

output_directory='/enigmaDTI/QC_ENIGMA/QC_FA_V1/';

thresh=0.2;

[subjs,FAs,VECs]=textread(TXTfile,'%s %s %s','headerlines',1)

for s = 1:length(subjs)

subj=subjs(s);

Fa=FAs(s);

Vec=VECs(s);

try

% reslice FA

[pathstrfa,nameniifa,gzfa] = fileparts(Fa{1,1});

[nafa,namefa,niifa] = fileparts(nameniifa);

newnamegzfa=[pathstrfa,'/',namefa,'_reslice.nii.gz'];

newnamefa=[pathstrfa,'/',namefa,'_reslice.nii'];

copyfile(Fa{1,1},newnamegzfa);

gunzip(newnamegzfa);

delete(newnamegzfa);

reslice_nii(newnamefa,newnamefa);

% reslice V1

[pathstrv1,nameniiv1,gzv1] = fileparts(Vec{1,1});

[nav1,namev1,niiv1] = fileparts(nameniiv1);

newnamegzv1=[pathstrv1,'/',namev1,'_reslice.nii.gz'];

newnamev1=[pathstrv1,'/',namev1,'_reslice.nii'];

copyfile(Vec{1,1},newnamegzv1);

gunzip(newnamegzv1);

delete(newnamegzv1);

reslice_nii(newnamev1,newnamev1);

% qc

func_QC_enigmaDTI_FA_V1(subj,newnamefa,newnamev1, output_directory);

close(1)

close(2)

close(3)

% delete

delete(newnamefa)

delete(newnamev1)

end

display(['Done with subject: ', num2str(s), ' of ', num2str(length(subjs))]);

end

For troubleshooting individual subjects that func_QC_enigmaDTI_FA_V1.m script can be run in the command console with the following parameters:

func_QC_enigmaDTI_FA_V1('subjectID', 'FA_image_path', 'V1_image_path','output_directory')

Step 4 - Make the QC webpage

Within a terminal session go to the /enigmaDTI/QC_ENIGMA/ directory where you stored the script make_enigmaDTI_FA_V1_QC_webpage.sh and ensure it is executable:

chmod 777 make_enigmaDTI_FA_V1_QC_webpage.sh

or for Mac,

chmod 777 make_enigmaDTI_FA_V1_QC_webpage_mac.sh

Run the script, specifying the full path to the directory where you stored the Matlab QC output files:

./make_enigmaDTI_FA_V1_QC_webpage.sh /enigmaDTI/QC_ENIGMA/QC_FA_V1/

or for Mac,

sh make_enigmaDTI_FA_V1_QC_webpage_mac.sh /enigmaDTI/QC_ENIGMA/QC_FA_V1/

This script will create a webpage called enigmaDTI_FA_V1_QC.html in the same folder as your QC output. To open the webpage in a browser in a Linux environment type:

firefox /enigmaDTI/QC_ENIGMA/QC_FA_V1/enigmaDTI_FA_V1_QC.html

Scroll through each set of images to check that the vector directions are correct. For closer inspection, clicking on a subject’s preview image will provide a larger image. If you want to check the segmentation on another computer, you can just copy over the whole /enigmaDTI/QC_ENIGMA/QC_FA_V1/ output folder to your computer and open the webpage from there.

Congrats! Now you should have all you need to make sure your FA images turned out OK and their vector fields are aligned!

ENIGMA-DTI Processing

ENIGMA-DTI Skeletonization

Protocols

- ENIGMA-TBSS protocol

- Once DTI data have been pre-processed, use the protocol above to map your images onto the ENIGMA-DTI FA template and project the skeleton.

- Do not forget to QC your images after FA maps have been registered (http://enigma.ini.usc.edu/wp-content/uploads/DTI_Protocols/ENIGMA_FA_Skel_QC_protocol_USC.pdf)! Misaligned images can cause problems with the overall GWAS and are difficult to detect once skeletonized.

Protocol for TBSS analysis

The following steps will allow you to register and skeletonize your FA images to the DTI atlas being used for ENIGMA-DTI for tract-based spatial statistics (TBSS; Smith et al., 2006).

Here we assume preprocessing steps including motion/Eddy current correction, masking, tensor calculation, and creation of FA maps has already been performed, along with quality control (http://enigma.ini.usc.edu/wpcontent/uploads/DTI_Protocols/ENIGMA_FA_V1_QC_protocol_USC.pdf).

Further instructions for using FSL, particularly TBSS can be found on the website:

http://www.fmrib.ox.ac.uk/fsl/tbss/index.html

- Download a copy of the ENIGMA-DTI template FA map, edited skeleton, masks and corresponding distance map from the following link into a directory (example /enigmaDTI/TBSS/ENIGMA_targets/)

http://enigma.ini.usc.edu/wp-content/uploads/2013/02/enigmaDTI.zip

The downloaded archive will have the following files:

  - ENIGMA_DTI_FA.nii.gz

  - ENIGMA_DTI_FA_mask.nii.gz

  - ENIGMA_DTI_FA_skeleton.nii.gz

  - ENIGMA_DTI_FA_skeleton_mask.nii.gz

  - ENIGMA_DTI_FA_skeleton_mask_dst.nii.gz

- Copy all FA images into a folder

cp /subject*_folder/subject*_FA.nii.gz /enigmaDTI/TBSS/run_tbss/

- cd into directory and erode images slightly with FSL

cd /enigmaDTI/TBSS/run_tbss/

tbss_1_preproc *.nii.gz

This will create a ./FA folder with all subjects eroded images and place all original ones in a ./origdata folder

- Register all subjects to ENIGMA_DTI_FA template

Can choose registration method that works best for your data

<<< as a default use ENIGMA >>>

4.1 First lets make sure the FOV is appropriate and mask the ENIGMA-DTI template

Copy all FA_FA images into a folder

mkdir /enigmaDTI/TBSS/run_tbss/FA2ENIGMAtemplate

cp /enigmaDTI/TBSS/run_tbss/FA/subject*_FA_FA.nii.gz /enigmaDTI/TBSS/run_tbss/FA2ENIGMAtemplate

4.2 Within a terminal session go to the directory and run the following script

for subj in subj_1 subj_2 … subj_N

do

flirt -in /enigmaDTI/TBSS/run_tbss/FA2ENIGMAtemplate/${subj}_FA_FA.nii.gz -ref ENIGMA_DTI_FA.nii.gz -out /enigmaDTI/TBSS/run_tbss/FA2ENIGMAtemplate/${subj}_FA_to_target.nii.gz -omat /enigmaDTI/TBSS/run_tbss/FA2ENIGMAtemplate/${subj}_FA_to_target.mat -dof 9

done

Make sure to QC images to ensure good registration!

4.3 Copy all FA_to_target images into a folder

cp /enigmaDTI/TBSS/run_tbss/FA2ENIGMAtemplate/subject*_FA_to_target.nii.gz /enigmaDTI/TBSS/run_tbss/QC_Reg

4.4 Within a terminal session go to the directory

/enigmaDTI/TBSS/run_tbss/QC_Reg

and run the following script to make the QC with FSLview:

#!/bin/sh

for s in `$FSLDIR/bin/imglob *`; do

echo ""

echo "###############################"

echo "checking subject ${s}"

echo "check registration in fslview and close when done"

echo "###############################"

echo ""

$FSLDIR/bin/fslview ${FSLDIR}/data/standard/FMRIB58_FA_1mm ${s} -l “Greyscale”

done

Example of bad registration is available at http://enigma.ini.usc.edu/wp-content/uploads/DTI_Protocols/eDTI_protocolFigures/1_bad_reg.png

<<< if any maps are poorly registered, move them to another folder >>>

mkdir /enigmaDTI/TBSS/run_tbss/BAD_ REGISTER/

mv FA_didnt_pass_QC* /enigmaDTI/TBSS/run_tbss/BAD_REGISTER/

**NOTE** If your field of view is different from the ENIGMA template – (example, you are missing some cerebellum/temporal lobe from your FOV) or you find that the ENIGMA mask is somewhat larger than your images, please follow Steps 5 and 6 to remask and recreate the distance map. Otherwise, continue to use the distance map provided***

- Make a new directory for your edited version:

mkdir /enigmaDTI/TBSS/ENIGMA_targets_edited/

- Create a common mask for the specific study and save as:

/enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA_mask.nii.gz

- One option to create a common mask for your study (in ENIGMA space) is to combine all well registered images and see where most subjects (here 90%) have brain tissue using FSL tools and commands:

cd /enigmaDTI/TBSS/run_tbss/

${FSLPATH}/fslmerge –t ./all_FA_QC ./FA2ENIGMAtemplate/*FA_to_target.nii.gz

${FSLPATH}/fslmaths ./all_FA_QC -bin -Tmean –thr 0.9 /enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA_mask.nii.gz

Mask and rename ENIGMA_DTI templates to get new files for running TBSS:

${FSLPATH}/fslmaths /enigmaDTI/TBSS/ENIGMA_targets/ENIGMA_DTI_FA.nii.gz –mas /enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA_mask.nii.gz

/enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA.nii.gz

${FSLPATH}/fslmaths /enigmaDTI/TBSS/ENIGMA_targets/ENIGMA_DTI_FA_skeleton.nii.gz –mas /enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA_mask.nii.gz /enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA_skeleton.nii.gz

Your folder should now contain:

/enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA.nii.gz

/enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA_mask.nii.gz

/enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA_skeleton.nii.gz

4.5 Now perform non-linear registration to masked template

<<< as a default use FSL >>>

tbss_2_reg –t /enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA.nii.gz

tbss_3_postreg -S

- cd into directory where you have newly masked ENIGMA target and skeleton to create a distance map

tbss_4_prestats -0.049

- The distance map will be created but the function will return an error because the all_FA is not included here. This is ok!

- The skeleton has already been thresholded here so we do not need to select a higher FA value (ex 0.2) to threshold.

will output: /enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA_skeleton_mask_dst

Your folder should now contain at least:

/enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA.nii.gz

/enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA_mask.nii.gz

/enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA_skeleton.nii.gz

/enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA_skeleton_mask.nii.gz

/enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA_skeleton_mask_dst.nii.gz

**NOTE** For the following steps, if you use the ENIGMA mask and distance map as provided, in the commands for steps 7 and 8 replace:

/enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA_mask.nii.gz with /enigmaDTI/TBSS/ENIGMA_targets/ENIGMA_DTI_FA.nii.gz

and

/enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA_skeleton_mask_dst with /enigmaDTI/TBSS/ENIGMA_targets/ENIGMA_DTI_FA_skeleton_mask_dst

***

- For faster processing or parallelization, it is helpful to run the projection on one subject at a time. Move each subject FA image into its own directory and (if masking was necessary as in steps 5 and 6 above) mask with common mask. This can be parallelized on a multiprocessor system if needed.

cd /enigmaDTI/TBSS/run_tbss/

for subj in subj_1 subj_2 … subj_N

do

mkdir -p ./FA_individ/${subj}/stats/

mkdir -p ./FA_individ/${subj}/FA/

cp ./FA/${subj}_*.nii.gz ./FA_individ/${subj}/FA/

####[optional/recommended]####

${FSLPATH}/fslmaths ./FA_individ/${subj}/FA/${subj}_*FA_to_target.nii.gz -mas /enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA_mask.nii.gz

./FA_individ/${subj}/FA/${subj}_masked_FA.nii.gz

done

- Skeletonize images by projecting the ENIGMA skeleton onto them:

9.1 Copy all FA_to_target images into a folder /enigmaDTI/TBSS/run_tbss/FA

cp /enigmaDTI/TBSS/run_tbss/FA2ENIGMAtemplate/subject*_FA_to_target.nii.gz /enigmaDTI/TBSS/run_tbss/FA

9.2 Within a terminal session go to the directory

cd /enigmaDTI/TBSS/run_tbss/

for subj in subj_1 subj_2 … subj_N

do

${FSLPATH}/tbss_skeleton -i ./FA_individ/${subj}/FA/${subj}_masked_FA.nii.gz -p 0.049  /enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA_skeleton_mask_dst ${FSLPATH}/data/standard/LowerCingulum_1mm.nii.gz ./FA_individ/${subj}/FA/${subj}_masked_FA.nii.gz ./FA_individ/${subj}/stats/${subj}_masked_FAskel.nii.gz -s /enigmaDTI/TBSS/ENIGMA_targets_edited/mean_FA_skeleton_mask.nii.gz

done

Congrats! Now you have all your images in the ENIGMA-DTI space with corresponding projections.

All your skeletons are:

/enigmaDTI/TBSS/run_tbss/FA_individ/${subj}/stats/${subj}_masked_FAskel.nii.gz

Protocol for FA and Skeleton Visual QC analysis for ENIGMA-DTI

The following steps will allow you to visualize your FA images after registration to the ENIGMA-DTI template, and to see if your extracted skeletons are all projected onto the ENIGMA Skeleton.

These protocols are offered with an unlimited license and without warranty. However, if you find these protocols useful in your research, please provide a link to the ENIGMA website in your work: www.enigma.ini.usc.edu

Highlighted portions of the instructions may require you to make changes so that the

commands work on your system and data.

Instructions

Prerequisites

• 	Matlab installed http://www.mathworks.com/products/matlab/

• 	Diffusion-weighted images preprocessed using FSL’s DTIFIT

(http://fsl.fmrib.ox.ac.uk/fsl/fsl4.0/fdt/fdt_dtifit.html) or equivalent.

• 	Run the ENIGMA DTI processing protocol to project individual skeletons onto the

common template: http://enigma.ini.usc.edu/protocols/dti-protocols/ - eDTI

Step 1 – Download the utility packages

Download the Matlab scripts package for Step 3:

http://enigma.ini.usc.edu/wp-content/uploads/DTI_Protocols/enigmaDTI_QC.zip

Download the script to build the QC webpage for Step 4:

• Linux: http://enigma.ini.usc.edu/wp-

content/uploads/DTI_Protocols/make_enigmaDTI_FA_Skel_QC_webpage.sh

• Mac: http://enigma.ini.usc.edu/wp-

content/uploads/DTI_Protocols/make_enigmaDTI_FA_Skel_QC_webpage_mac.sh

Step 2 – Build a text file defining the location of subject files

Create a three column tab-delimited text file (e.g. Subject_Path_Info.txt):

• subjectID: subject ID

• FAimage: full path to registered FA image.

• Skeleton: full path to skeletonized FA image.

subjectID			 FAimage 					Skeleton

USC_01			 /path/USC_01_ masked_FA.nii.gz 		/path/USC_01_ masked_FAskel.nii.gz

USC_02 			/path/USC_02_ masked_FA.nii.gz 		/path/USC_02_ masked_FAskel.nii.gz

USC_03 			/path/USC_03_ masked_FA.nii.gz 		/path/USC_03_ masked_FAskel.nii.gz

Step 3 – Run Matlab script to make QC images

Unzip the Matlab scripts from Step 1 and change directories to that folder with the required Matlab *.m scripts. For simplicity, we assume you are working on a Linux machine with the base directory /enigmaDTI/QC_ENIGMA/.

Make a directory to store all of the QC output:

mkdir /enigmaDTI/QC_ENIGMA/QC_FA_SKEL/

Start Matlab:

/usr/local/matlab/bin/matlab

Next we will run the func_QC_enigmaDTI_FA_skel.m script that reads the Subject_Path_Info.txt file to create subdirectories in a specified output_directory for each individual subjectID, then create an axial, coronal and sagittal image of the FA_image with overlays from the Skeleton.

In the Matlab command window paste and run:

TXTfile='/enigmaDTI/QC_ENIGMA/Subject_Path_Info.txt';

output_directory='/enigmaDTI/QC_ENIGMA/QC_FA_SKEL/';

[subjs,FAs,SKELs]=textread(TXTfile,'%s %s %s','headerlines',1)

for s = 1:length(subjs)

subj=subjs(s);

Fa=FAs(s);

skel=SKELs(s);

try

% reslice FA

[pathstrfa,nameniifa,gzfa] = fileparts(Fa{1,1}); [nafa,namefa,niifa] = fileparts(nameniifa); newnamegzfa=[pathstrfa,'/',namefa,'_reslice.nii.gz']; newnamefa=[pathstrfa,'/',namefa,'_reslice.nii']; copyfile(Fa{1,1},newnamegzfa);

gunzip(newnamegzfa); delete(newnamegzfa); reslice_nii(newnamefa,newnamefa);

% reslice skel

[pathstrskel,nameniiskel,gzskel] = fileparts(skel{1,1}); [naskel,nameskel,niiskel] = fileparts(nameniiskel); newnamegzskel =[pathstrskel,'/',nameskel,'_reslice.nii.gz']; newnameskel =[pathstrskel,'/',nameskel,'_reslice.nii']; copyfile(skel{1,1},newnamegzskel);

gunzip(newnamegzskel);

delete(newnamegzskel);

reslice_nii(newnameskel,newnameskel);

% qc

func_QC_enigmaDTI_FA_skel(subj,newnamefa,newnameskel, output_directory);

￼￼￼￼/enigmaDTI/QC_ENIGMA/Subject_Path_Info.txt

/enigmaDTI/QC_ENIGMA/QC_FA_SKEL/

close(1)

close(2)

close(3)

% delete

delete(newnamefa)

delete(newnameskel)

end

display(['Done with subject: ', num2str(s), ' of ', num2str(length(subjs))]);

end

For troubleshooting individual subjects func_QC_enigmaDTI_FA_skel.m script can be run in the command console with the following parameters:

func_QC_enigmaDTI_FA_skel('subjectID', 'FA_image_path', 'Skel_image_path','output_directory')

Step 4 - Make the QC webpage

Within a terminal session go to the /enigmaDTI/QC_ENIGMA/ directory where you stored the script make_enigmaDTI_FA_Skel_QC_webpage.sh and ensure it is executable:

chmod 777 make_enigmaDTI_FA_Skel_QC_webpage.sh

or for Mac,

chmod 777 make_enigmaDTI_FA_Skel_QC_webpage_mac.sh

Run the script, specifying the full path to the directory where you stored the Matlab QC output files:

./make_enigmaDTI_FA_Skel_QC_webpage.sh /enigmaDTI/QC_ENIGMA/QC_FA_SKEL/

or for Mac,

sh make_enigmaDTI_FA_Skel_QC_webpage_mac.sh /enigmaDTI/QC_ENIGMA/QC_FA_SKEL/

This script will create a webpage called enigmaDTI_FA_Skel_QC.html in the same folder as

your QC output. To open the webpage in a browser in a Linux environment type:

firefox /enigmaDTI/QC_ENIGMA/QC_FA_SKEL/enigmaDTI_FA_Skel_QC.html

Scroll through each set of images to check that the images are all aligned and well registered and all skeletons are composed of the same voxels. For closer inspection, clicking on a subject’s preview image will provide a larger image. If you want to check the segmentation on another

computer, you can just copy over the whole /enigmaDTI/QC_ENIGMA/QC_FA_SKEL/ output folder to your computer and open the webpage from there.

Congrats! Now you should have all you need to make sure your FA images turned out OK and their skeletons line up.

Use this script after skeletonizing your FA images to check the mean and max projection distances to the skeleton:

#!/bin/sh

# Emma Sprooten for ENIGMA-DTI

# run in a new directory eg. Proj_Dist/

# create a text file containing paths to your masked FA maps

# output in Proj_Dist.txt

# make sure you have FSL5!!!

###### USER INPUTS ###############

## insert main folder where you ran TBSS

## just above "stats/" and "FA/"

maindir="/enigmaDTI/TBSS/run_tbss/"

list=`find $maindir -wholename "*/FA/*_masked_FA.nii.gz"`

## insert full path to mean_FA, skeleton mask and distance map

## based on ENIGMA-DTI protocol this should be:

mean_FA="/enigmaDTI/TBSS/ENIGMA_targets/mean_FA.nii.gz"

mask="/enigmaDTI/TBSS/ENIGMA_targets/mean_FA_skeleton_mask.nii.gz"

dst_map="/enigmaDTI/TBSS/ENIGMA_targets/enigma_skeleton_mask_dst.nii.gz"

##############

### from here it should be working without further adjustments

rm Proj_Dist.txt

echo "ID" "Mean_Squared" "Max_Squared" >> Proj_Dist.txt

## for each FA map

for FAmap in ${list}

do

base=`echo $FAmap | awk 'BEGIN {FS="/"}; {print $NF}' | awk 'BEGIN {FS="_"}; {print $1}'`

dst_out="dst_vals_"$base""

# get Proj Dist images

tbss_skeleton -d -i $mean_FA -p 0.2 $dst_map $FSLDIR/data/standard/LowerCingulum_1mm $FAmap $dst_out

#X direction

Xout=""squared_X_"$base"

file=""$dst_out"_search_X.nii.gz"

fslmaths $file -mul $file $Xout

#Y direction

Yout=""squared_Y_"$base"

file=""$dst_out"_search_Y.nii.gz"

fslmaths $file -mul $file $Yout

#Z direction

Zout=""squared_Z_"$base"

file=""$dst_out"_search_Z.nii.gz"

fslmaths $file -mul $file $Zout

#Overall displacement

Tout="Total_ProjDist_"$base""

fslmaths $Xout -add $Yout -add $Zout $Tout

# store extracted distances

mean=`fslstats -t $Tout -k $mask -m`

max=`fslstats -t $Tout -R | awk '{print $2}'`

echo "$base $mean $max" &gt;&gt; Proj_Dist.txt

# remove X Y Z images

## comment out for debugging

rm ./dst_vals_*.nii.gz

rm ./squared_*.nii.gz

echo "file $Tout done"

done

ROI extraction from FA images

- ENIGMA-ROI extraction protocol (PDF format)

- ENIGMA-ROI extraction protocol (Word format)

Use the above link to extract regions of interest from the skeletons and calculate the average FA within them.

**** An important note **** in the analysis is that the ROI labeled “IFO” is actually different that the same ROI under the most current FSL JHU Atlas label. This was different in the older JHU atlas used to create the template and make the protocols. Please note this will not play a role in ENIGMA-DTI GWAS as we will not be using this ROI for GWAS (it is very small) but it may be considered for disorder studies.

If groups choose to use these measures for their own analysis, please be advised that this should be uncinate according to the current atlas. To avoid confusion, we will NOT switch the labels back, but we will keep this warning so you can carefully examine your data.

Protocol for ROI analysis using the ENIGMA-DTI template

The following steps will allow you to extract relevant ROI information from the skeletonized FA images that have been registered and skeletonized according to the ENIGMA-DTI template, and keep track of them in a spreadsheet.

Here we assume that you have a common meta-data spreadsheet with all relevant covariate information for each subject.

• Can be a tab-delimited text file, or a .csv

• Ex) MetaDataSpreadsheetFile.csv :

• The following is an example of a data spreadsheet with all variables of interest. This spreadsheet is something you may already have to keep track of all subject information. It will be used later to extract only information of interest in Step 6

• An example file is provided – ALL_Subject_Info.txt

INSTRUCTIONS

1. Download and install ‘R’ http://cran.r-project.org/

2. Download a copy of the scripts and executables here:

• http://enigma.ini.usc.edu/wp-content/uploads/2012/06/ROIextraction_info.zip

Bash shell scripts and compiled versions of the code (bold) have been made available to run on Linux -based workstations. Raw code is also provided in the case re-compilation is needed.

The downloaded archive will have the following files:

• run_ENIGMA_ROI_ALL_script.sh

• singleSubjROI_exe

• singleSubject_FA_ROI.cpp

• averageSubjectTracts_exe

• average_subj_tract_info.cpp

• run_combineSubjectROI_script.sh

• combine_subject_tables.R

necessary files --

• ENIGMA_look_up_table.txt

• JHU-WhiteMatter-labels-1mm.nii.gz

• mean_FA_skeleton.nii.gz

example files --

• ALL_Subject_Info.txt

• subjectList.csv

• Subject1_FAskel.nii.gz

• Subject7_FAskel.nii.gz

example outputs --

• Subject1_ROIout.csv

• Subject1_ROIout_avgs.csv

• Subject7_ROIout.csv

• Subject7_ROIout_avgs.csv

• combinedROItable.csv

3. run_ENIGMA_ROI_ALL_script.sh provides an example shell script on how to run all the pieces in series.

• This can be modified to run the first two portions in parallel if desired.

- The first command - singleSubjROI_exe uses the atlas and skeleton to extract ROI values from the JHU-atlas ROIs as well as an average FA value across the entire skeleton

• It is run with the following inputs

• ./singleSubjROI_exe look_up_table.txt skeleton.nii.gz JHU-WhiteMatterlabels-1mm.nii.gz OutputfileName Subject_FA_skel.nii.gz

• example -- ./singleSubjROI_exe ENIGMA_look_up_table.txt mean_FA_skeleton.nii.gz JHU-WhiteMatter-labels-1mm.nii.gz Subject1_ROIout Subject1_FAskel.nii.gz

• The output will be a .csv file called Subject1_ROIout.csv with all mean FA values of ROIs listed in the first column and the number of voxels each ROI contains in the second column (see ENIGMA_ROI_part1/Subject1_ROIout.csv for example output)

- The second command - averageSubjectTracts_exe uses the information from the first output to average relevant (example average of L and R external capsule) regions to get an average value weighted by volumes of the regions.

• It is run with the following inputs

• ./averageSubjectTracts_exe inSubjectROIfile.csv outSubjectROIfile_avg.csv

• where the first input is the ROI file obtained from Step a) and the second input is the name of the desired output file.

• The output will be a .csv file called outSubjectROIfile_avg.csv with all mean FA values of the new ROIs listed in the first column and the number of voxels each ROI contains in the second column (see ENIGMA_ROI_part2/Subject1_ROIout_avg.csv for example output)

- The final portion of this analysis is an ‘R’ script combine_subject_tables.R that takes into account all ROI files and creates a spreadsheet which can be used for GWAS or other association tests. It matches desired subjects to a meta-data spreadsheet, adds in desired covariates, and combines any or all desired ROIs from the individual subject files into individual columns.

• Input arguments as shown in the bash script are as follows:

- Table=./ALL_Subject_Info.txt –

- A meta-data spreadsheet file with all subject information and any and all covariates

- subjectIDcol=subjectID

- the header of the column in the meta-data spreadsheet referring to the subject IDs so that they can be matched up accordingly with the ROI files

- subjectList=./subjectList.csv

- a two column list of subjects and ROI file paths.

- this can be created automatically when creating the average ROI .csv files – see run_ENIGMA_ROI_ALL_script.sh on how that can be done

- outTable=./combinedROItable.csv

- the filename of the desired output file containing all covariates and ROIs of interest

- Ncov=2

- The number of covariates to be included from the meta-data spreadsheet

- At least age and sex are recommended

- covariates="Age;Sex"

- the column headers of the covariates of interest

- these should be separated by a semi-colon ‘;’ and no spaces

- Nroi="all" #2

- The number of ROIs to include

- Can specify “all” in which case all ROIs in the file will be added to the spreadsheet

- Or can specify only a certain number, for example 2 and write out the 2 ROIs of interest in the next input

- rois= “all” #"IC;EC"

- the ROIs to be included from the individual subject files

- this can be “all” if the above input is “all”

- or if only a select number (ex, 2) ROIs are desired, then the names of the specific ROIs as listed in the first column of the ROI file

• these ROI names should be separated by a semi-colon‘;’ and no spaces for example if Nroi=2, rois="IC;EC" to get only information for the internal and external capsules into the output .csv file

• (see combinedROItable.csv for example output)

Congrats! Now you should have all of your subjects ROIs in one spreadsheet with only

relevant covariates ready for association testing!

Protocol for Creating Histograms and Summary Stats QC analysis for ENIGMA-DTI

The following steps will allow you to visualize your final FA distribution in each ROI in the form of a histogram and will output a text file with summary statistics on each ROI including the mean, standard deviation, min and max value, as well as the subjects corresponding to the min and max values.

These protocols are offered with an unlimited license and without warranty. However, if you find these protocols useful in your research, please provide a link to the ENIGMA website in your work: www.enigma.ini.usc.edu

Generate Summary Statistics and Histogram Plots

Highlighted portions of the instructions may require you to make changes so that the commands work on your system and data.

This section assumes that you have installed:

R (download here)

Download the automated script for generating the plots (called	ENIGMA_DTI_plots_ALL.R)

http://enigma.ini.usc.edu/wp-content/uploads/DTI_Protocols/ENIGMA_DTI_plots_ALL.R

After having quality checked each of your segmented structures you should have a file called combinedROItable.csv, which is a comma separated file with the mean FA of each ROI for each subject.

It should look like this (note the ... there should be 64 + however many covariates of interest columns):

"subjectID","Age","Sex","ACR","ACR-L","ACR-R","ALIC","ALIC-L","ALIC-R","AverageFA","BCC",

... subject1,

... subject2,

... subject3,

...

Generating plots and summary statistics:

Make a new directory to store necessary files:

mkdir /enigmaDTI/figures/

Copy your combinedROItable.csv file to your new folder:

cp /enigmaDTI/combinedROItable.csv / enigmaDTI/figures/

Move the ENIGMA_DTI_plots.R script to the same folder:

mv /enigmaDTI/downloads/ENIGMA_DTI_plots.R /enigmaDTI/figures/

Make sure you are in your new figures folder:

cd /enigmaDTI/figures

The code will make a new directory to store all of your summary stats and histogram plots:

/enigmaDTI/figures/QC_ENIGMA

Run the R script to generate the plots, make sure to enter your cohort name so it shows up on all plots:

cohort= 'MyCohort'

R --no-save --slave --args ${cohort} < ENIGMA_DTI_plots_ALL.R

It should only take a few minutes to generate all of the plots. If you get errors, the script might tell you what things need to be changed in your data file in order to work properly. Just make sure that your input file is in *.csv format similar to the file above.

The output will be a pdf file with a series of histograms. You need to go through each page to make sure that your histograms look approximately normal. If there appear to be any outliers, please verify your original FA image is appropriate. If you end up deciding that certain subjects are have poor quality scans then you should give that subject an “NA” for all ROIs in your combinedROItable.csv file and then re-run the ENIGMA_DTI_plots_ALL.R script given above.

Please upload the ENIGMA_DTI_allROI_histograms.pdf and the ENIGMA_DTI_allROI_stats.txt files to your corresponding ENIGMA_DTI coordinator!